�����������������������������������������������ͻ

�     WAYPOINT LIST - AMVER MESSAGE PROGRAM     �

� Version 3.1 corrected  (c) 1990 by Ron Ristad �

� Revised/Compiled by Richard M. Plant 03/22/90 �

�����������������������������������������������ͼ

The first 4 pages of the documentation are for those who

do not like to read documents.  The remaining pages go into

more detail on how the program works.  Please read the

CAUTION found on PAGE 3.  Thank you.

The AMVER program, written by Ron Ristad has been

significantly modified and improved by Richard M. Plant

* * *  with the issue date being 3/22/90.  The waypoint limit

has been increased to 61 points.  Please take note

that the NEW version is compiled and has color added

to it enhancing its operation by making it more user

friendly, increasing its speed tremendously and

protecting the program from changes.

The PARAMETER list (AMVER.PRM) is new in that the users

SHIPPING COMPANY can now be added and data disk from A:

to F: works.  If more drives are needed please let me

know.  Also, the subdirectory for the EMAIL drive can now

be specified thus allowing the use of this program on any

hard disk or LAN system.

You can quit to DOS rather then SHELLING to DOS leaving

the AMVER program resident in memory.  You achieve this

by getting back to the main menu and selecting Q (Quit to

DOS).  Also, from the waypoint menu you can return to the

Main Menu and back again.  This allows you to check the

program setup without exiting and reentering the program.

As you can see the description of the program is now

OUTSIDE of the program.  The adding of color and

compiling of the program added to the memory such that I

am right now in the process of reviewing the program to

see what can be eliminated so I can increase its

capability.  Six (6) noon slips per printed page can now

be printed and the prompting of information and the error

messages in many cases are a little more descriptive.

CAUTION: Upon running AMVER for the first time please

P - Program set-up

and confirm that all items, especially the disk drives,

are set up properly.  Drives A: thru F: are permitted.

If you need more than please let me know.  Remember, you

can now define the EMAIL subdirectory.  Also, the RPM

limit has been increased from 105 to 150 RPM's.  If more

are needed please let me know.

PAGE 1

WAYPOINT ENTRY

By entering the latitude and longitude of two or more

waypoints (entering just one waypoint is not allowed

any more), this program will calculate the distances

between waypoints, total distance, and courses to steer

for both rhumbline and/or great circle legs.  It operates

as a spreadsheet, automatically calculating the ETA at

each waypoint based on the estimated speed or computed

speed, or the speed/rpm's required based on a required

arrival time.

Waypoint lists for frequently traveled routes may be

saved as permanent disk files, needing only to call up

the file to print out the waypoint list and a chart list

for the voyage, and needing only to plug in the departure

and required arrival times (or time of departure and

estimated speed) in order to compose an AMVER departure

message.  A complete set of editing commands allows

waypoint lists to be easily changed or modified for

weather routing, etc. AND NOW all this is done from the

waypoint list screen.

By entering noon positions during the course of a voyage,

the program will make the necessary calculations and

print out noon slips and compose AMVER position reports.

Each time a fix is entered, the program will calculate

the current speed, and revise  ETA's at each waypoint

based on this speed...Or, if there is a required arrival

time, then it will calculate a new required speed, thus

allowing for maximum fuel economy.  It also calculates

the new course to steer from the present position.  If

there are course diversions, due to weather, etc., it

will print out a revised waypoint list and generate an

AMVER deviation message.  At the end of the voyage it

will compose an AMVER arrival message.  AMVER messages

may be sent to a printer or saved to an Email disk and

subdirectory as you specified in the program set up.

GREAT CIRCLE LEGS

The program quickly and easily computes points at

intervals along great circle tracks, as well as the

rhumbline courses and distances between points for

plotting great circle tracks on mercator charts and

plotting sheets.  Great Circle intervals of whole degrees

from 1 to 10 degrees are presently acceptable.  At

present the first position calculated might not be your

desired interval from the departure point.  The program

finds the first point that is evenly divisible by your

interval from the departure point and then uses your

entered interval value to calculate all subsequent

points.

PAGE 2

DISK FILES

Two types of files are used, permanent and working files.

PERMANENT      contain only the latitude, longitude,

waypoint labels, and tracks (coastal,

rhumbline, or great circle) between

waypoints.

WORKING        are used during the course of the voyage

and contain departure time, ETA's and

required arrival time(s), speeds, fixes,

noon positions, etc..  Make sure that you

save all working files after any fix or

new arrival time has been entered.

FILE NAMES are always 7 or 8 characters in length.  A

Keelung-to-Kaoshiung permanent file would be named

KEE-KAO, with alternate routes being named KEE-KAO1,

KEE-KAO2, etc..  Regardless of which route is chosen, the

working file for the voyage would be named KEE-KAOW.

Using this file naming convention keeps the disk from

getting filled-up and makes it easy to remember file

names.

���������ͻ

����������������������͹ CAUTION �����������������������ͻ

�                      ���������ͼ                       �

�  Please take note that the HELP file utilizes a SHELL  �

�  to DOS.  This requires that either the COMMAND.COM    �

�  file, for the DOS you booted the computer with, be on �

�  the AMVER program disk or that the path to the file   �

�  COMMAND.COM be located in the path set up found in    �

�  in the AUTOEXEC.BAT file or that the COMSPECification �

�  be set in the CONFIG.SYS file.                        �

�                                                        �

�    AUTOEXEC.BAT file     PATH = C:\                    �

�                                                        �

�    CONFIG.SYS file       SET COMSPEC=C:\COMMAND.COM    �

�                                                        �

�  Without either of these two commands or without the   �

�  COMMAND.COM file being on the AMVER program disk you  �

�  will find the program crashing when ever you try to   �

�  access the HELP feature and View the file.            �

�                                                        �

�  Please see either me or another knowledgable person   �

�  regarding the setting up of the computer to allow     �

�  access to the COMMAND.COM file for SHELLing.          �

�                                                        �

�  This same requirement existed in the original program �

�  by RON RISTAD but was never explained.                �

�                                                        �

��������������������������������������������������������ͼ

PAGE 3

DATA ENTRY FORMATS

LATITUDES AND LONGITUDES are entered with a hyphen (-) or

decimal  point (.) separating degrees and minutes.

Hyphens or decimal points are not required to input whole

degrees.

Examples: 22-00.3N  =  22 degrees  00.3 minutes North

25N  =  25 degrees  00.0 minutes North

1.25.8E  =   1 degree   25.8 minutes East

DATE/TIME is always entered in GMT as a 6 digit number

in the format of DDHHMM.  121800 =  12th 1800

GMT (The one exeception to this is the local

ship's date which appears on the noon slip,

which may be written in any fashion, e.g.,

9-12-87, 9/12/87, etc.. with no commas).

TRACKS    Only the first letter of the track need be

entered,

R = Rhumbline  G = Great circle  C = Coastal

Unless otherwise indicated in the program, you may exit

any of the routines by hitting the <ENTER> key when being

prompted for input.  When in an editing routine hitting

the <ENTER> key without typing any data will leave the

entry unchanged.  To delete an optional entry (names,

arrival/departure times, chart lists, etc.), enter a

slash mark (/).

Due to the many changes I would appreciate being notified

of any major changes needed or bugs in the system.  Thank

you and if you run across Ron Ristad - thank him.  He did

one hell of a job in creating this program.  All I tried

to do was make it more user friendly, colorful and

faster.

Richard M. Plant

M.I.T.A.G.S.

5700 Hammonds Ferry Road

Linthicum, MD 21090

301-859-5700

PAGE 4

03-10-1990                                          15:22:04

�����������������������������������������������ͻ

�     WAYPOINT LIST - AMVER MESSAGE PROGRAM     �

� Version 3.1 corrected  (c) 1990 by Ron Ristad �

� Revised/Compiled by Richard M. Plant 03/22/90 �

�����������������������������������������������ͼ

P - Program set-up

S - SET DATE/TIME

H - HELP

L - Load/delete waypoint file (d:)

N - New waypoint file

G - Great circle points

Q - Quit - DOS PROMPT

X - eXit - SHELL TO DOS

W - Waypoint file menu

P    PROGRAM SET-UP

You must enter the following permanent data as it

pertains to your ship.  This includes the company's name,

the ship's name and call sign, AMVER information, the

ship's sea speed and corresponding RPM's, and if the ship

is diesel-powered with an engine slow-down program enter

the slow-down schedule (present range is 90 to 40 RPM's).

The AMVER information is the radio coast station guarded,

which you will need to get from the Radio Officer, and

may need to be changed as the ship moves from one place

to another.

To delete all remaining data in a category simply enter

a slash (/) key.

The speed and corresponding RPM's are used to calculate

prop pitch. "@ zero slip" here means under conditions of

no wind and current, and may be different than published

values for the ship.  These values are best determined by

speed checks going in opposite courses at the same RPM

over the same area in calm weather.  Max of 150 RPM's.

If using this program on a hard disk or for an LAN system

make sure that the drive letters are correct.  Check with

a knowledgable officer or LAN administrator.  This

version of the program has been specifically altered to

run on the Local Area Network and any hard disk from

drive A: to F:.  If you change the drive letters to

incorrect or nonexistant drives, you will not be able to

load waypoint files or send AMVER messages to Email.  This

version ONLY has been modified to allow the data disk to

be other than the program disk!  Remember that you specify

the EMAIL subdirectory in the program set-up.

After entering data to conform to your ship, press Y to

save the data to a permanent file (AMVER.PRM).

PAGE 5

S    SET DATE/TIME.

Date and time are displayed at the top of the screen.

Press S to change date/time as required.  While running

the program the date and time will be checked to see if

they are valid for the operation being performed.  You

will be taken to the set up routine if invalid.

BEGINNING THE VOYAGE

99% of the time it's easier to load a permanent file for

the voyage, if one is on the disk, than it is to create

a new file.  Even if you delete all the waypoints but the

departure and arrival positions, you can use the insert

routine to enter the waypoints, and it will save you from

reentering the chart list and comments.

PERMANENT FILES - CREATING A NEW FILE

If you don't have a permanent file for the voyage, then

you must create a file.  After entering the names of the

departure and arrival ports for each waypoint you will

enter the latitude, longitude, waypoint label (optional),

and the track (rhumb line, great circle, or coastal).  If

the track for the leg is coastal, then you must also

enter the distance.  These are the only waypoint entries

you want to enter into the permanent file.  When the

routines ask for time and estimated speed, hit the

<ENTER> key to skip them.

After all the waypoints are entered, the next step is to

enter the chart list and any comments, using the main menu

"C" routine.  I usually type the name of the file under

comments, and keep a print-out of each file for a quick

reference of the different routes.

The final step is to save the permanent file.  In this

example we'll use a Keelung to Kaohsiung voyage and name

the file "KEE-KAO". Alternative routes would be named

"KEE-KAO1, KEE-KAO2", etc.

H    HELP

You are provided with the option of viewing to screen using

Vernon Buerg's LIST program, Version 7.2a OR printing the

the README.DOC file via the DOS COMMAND as follows:

TYPE README.DOC > PRN

Please read the CAUTION on page 3.

PAGE 6

L    LOAD/DELETE WAYPOINT FILE (d:)

Displays the waypoint file directory on the screen.  Note

that you may omit the ".DAT" file name extension when

entering the name of the waypoint file you want to load.

* * *  In this latest version, please note that entering the

word DELETE, in caps, for the filename allows you to

enter the filename you wish to delete.  BE CAREFUL !

THE .DAT NEED NOT BE ENTERED & WILDCARDS (* ?) DO WORK !

N    CREATE NEW WAYPOINT FILE

Used for making up new waypoint lists.  If you already

have a file for the route, it is easier to load the

existing file and use the editing routine to make the

necessary changes.  This save you time in not having to

make up another chart list, etc.

G    GREAT CIRCLE POINTS

Used as an aid in transferring great circle tracks to

mercator charts.  Computes latitudes for DLO intregal in

whole degrees of from 1 to 10 degree difference of

longitudes along a great circle track.  Please note that

the value entered for the increment might not be used as

the DLO interval to the first waypoint.  The program

calculates the first longitude so that all succeeding

points will be equally divisible by the desired DLO

interval entered.

Q    QUIT - DOS PROMPT

Terminates the program, removes it from memory and

returns you to the DOS or LAN menu system.  You will be

prompted    QUIT TO DOS (Y/N) ?    as a safety feature.

X    EXIT - SHELL TO DOS

Temporarily exits (SHELL) to DOS.  This feature allows

copying of waypoint files stored on floppies to your data

drive, deleting unwanted waypoint files from the data

drive and the execution of other necessary DOS commands.

To re-enter the AMVER program, you must type EXIT and

press ENTER.  Reloading AMVER by entering AMVER will take

up memory and eventually lock up your computer.

W    WAYPOINT FILE MENU

Once a file has been created you will no longer be on the

MAIN MENU but on the WAYPOINT FILE MENU.  To return to the

MAIN MENU use (M)enu from the WAYPOINT FILE MENU.  To return

to the WAYPOINT FILE MENU use W from the MAIN MENU.

PAGE 7

FILE NAME: SPE-YOKW

03-10-1990                18 Waypoints                     15:05:20

A - Arrival time          03 17:00Z   O - Find slow-down waypoints

B - Print waypoint list               P - Position Report    08 12:00Z

C - Chart list & comments             Q - Quit to MAIN MENU  09 12:00Z

D - Departure time/speed  03 02:00Z   R - RPM required

G - Great Circle Points               S - Save file

H - HELP                              T - Auto-track  OFF

J - Insert pre-arrival position       U - Engine slow-down timer

K - AMVER arrival message  E:\IMM     V - AMVER deviation message

L - Load/delete WP file    E:         W - WAYPOINT LIST

M - AMVER position report             Y - DR position

N - Create New waypoint file          Z - Print Position Report

The above menu items have been altered from the original

file to allow an easier handling of the waypoints.   You

should note that the E-Edit waypoint, F-View waypoint, and

I-Insert waypoint from the left side have been removed.

The W-Waypoint Arrival Time has been change to W-WAYPOINT

LIST and the X-Delete Waypoint has been removed.

As you will see in a few moments the functions removed have

all been incorporated at the bottom of the waypoint list as

follows:

Edit Delete Insert WP-Arrival time MENU Create WP#   # Cursor # PgDn PgUp

�    �      �      �               �    �            �        � ���� ����

The remaining pages explains all of the functions as shown

on both the WAYPOINT MENU and WAYPOINT LIST screens.

Please note that certian of the functions listed on the bottom

of the waypoint list are not always displayed.  Insert and

Create are available as long as the number of waypoints is

less than 60.  Cursor up, down and Page Up, down appear when

the numbe rof waypoints exceeds 15.  The last waypoint will

always appear on the bottom of the list when the waypoint count

exceeds 15.

PAGE 8

WAYPOINT FILE MENU

A    ARRIVAL TIME               DD HH:MMZ

These routines speed entry of arrival/departure

times.  In order to start a new working file

for the voyage, and print out an AMVER departure

message, you need only to:

1.   Load the permanent file for the selected

route.

2.   Enter date/time/month of departure and

estimated speed.

If there is a required time of arrival, then you

need only to:

1.   Load permanent file for the selected route.

2.   Enter required arrival time.

3.   Enter the date/time/month of departure

(estimated speed not required).

In cases where the calculated required speed is

less than the minimum desired speed for the vessel,

see "J - Insert pre-arrival position" routine.  The

Insert, Delete, and Edit routines are useful when

there is weather routing, as you need only load the

file for the usual route and make the necessary

changes, instead of making up a whole new file and

chart list.

B    PRINT WAYPOINT LIST

Prints a waypoint list showing waypoint number,

latitude, longitude, track, course, speed, distance

between waypoints and total distance from departure

as well as ETA.  Waypoint label is on the second line.

Route comments are printed on the bottom of the form.

This printout is more organized than the original

printout utilizing only 2 lines, instead of 3, per

waypoint with a space in between for clarity.

C    CHART LIST & COMMENTS

Used to add or view chart list or comments regarding

the waypoint list.  This information does not appear

in the waypoint list displayed on the screen.

PAGE 9

D    DEPARTURE TIME/SPEED   DD HH:MMZ

Enter the departure time (GMT) in the format DDHHMM

and the desired speed for the voyage.

G    GREAT CIRCLE POINTS

Used as an aid in transferring great circle tracks

to mercator charts.  Computes latitudes for DLO

intregal in whole degrees of from 1 to 10 degree

difference of longitudes along a great circle

track.  Please note that the value entered for the

increment might not be used as the DLO interval to

the first waypoint.  The program calculates the

first longitude so that all succeeding points will

be equally divisible by the desired DLO interval

entered.

Has options of automatically taking departure and

arrival points from great circle legs on the

waypoint list, or manually entering any departure

and arrival point.  Another option will put the

points in waypoint list form, showing the rhumb

line courses and distances between points, as well

as ETA at each point, if desired.  This routine

also allows the navigator to quickly and easily

plot a great circle course at any time from his

current position so the ship need not waste time

and fuel inorder to "get back on the course line."

H    HELP

You are provided with the option of viewing to screen

using Vernon Buerg's LIST program, Version 7.2a OR

printing the README.DOC file via the DOS COMMAND

as follows:

TYPE README.DOC > PRN

Please read the CAUTION on page 3.

PAGE 10

J    INSERT PRE-ARRIVAL POSITION

This routine calculates a waypoint along the voyage

track any number of miles prior to arrival and puts

it into the waypoint list.

A special use of this routine is for making up

accurate AMVER departure messages.  For example,

suppose that upon departure you enter a departure

time and a required arrival time, and the required

speed calculated is 18 knots.  However, your ship

has a high efficiency diesel engine that must be

run at a minimum speed of 20 knots. (In practice

these ships will often arrive early near their

destinations then stop engines and drift to

kill time.)

If you were to send an AMVER departure message

showing a speed of 18 knots, this would make the

AMVER plot inaccurate.  On the other hand, if you

were to use a speed of 20 knots, then the average

speed (AMVER "F" line) would be incorrect and could

also make the AMVER plot inaccurate.

Under these circumstances you may still compose an

accurate AMVER departure message using the

following sequence of steps exactly as follows:

1.   Load the permanent file for the chosen route.

2.   Use the "D - Departure time/speed" routine to

enter the time of departure and an estimated

speed of 20 knots (the minimum estimated

speed).

3.   Go to this routine and chose a distance a few

miles from arrival which is approximately the

distance you will start slowing down prior to

arrival.

4.   When this routine asks if this is for an AMVER

message speed adjustment type "Y", then enter.

5.   You will now be able to print out an accurate

AMVER departure message.

Another use of this routine is to factor in the

effect of slowing down the ship prior to arrival if

you are using the program to calculate daily noon

slips, as the routines for calculating the exact

slow-down positions are based on current conditions

and cannot be saved.

PAGE 11

Please note that the SAVE option is unavailable

after this option is selected.  You should first

save your file prior to executing this function.

For example, from experience you know that you should

start your slow-down approximately 17 miles or 1 hour,

15 minutes prior to arrival.  Supposing our required

arrival time was (DDHHMM) 100500 GMT, using this

routine we would:

1.   Enter a distance of 17 miles.

2.   When asked if this were for an AMVER message

speed adjustment, type "N".

3.   When asked for the required arrival time,

enter 100345 (1 hour, 15 minutes prior to

required arrival time).

4.   This will calculate and enter into the

waypoint list an approximate slow-down time

and position which can be saved, and which

will reflect a more accurate "speed required"

on the noon slip.

K    AMVER ARRIVAL MESSAGE      d:

Composes AMVER arrival message.  Allows you to Print,

Save or send to Email disk file.

L    LOAD/DELETE WAYPOINT FILE  d:

List all of the data files ending with the

extension .DAT found on the DATA disk as specified

in the program setup from the MAIN MENU.  The

entering of the extension is not necessary.  Files

ending with a W imply working files containing

departure time, arrival time, speed and have ETA

calculated.  Enter DELETE, in caps, for the filename

to allow you to enter the file to delete.  BE CAREFUL!

THE .DAT NEED NOT BE ENTERED & WILDCARDS (* ?) DO

WORK !  On an LAN you might be restricted to only READ

capability.  See the LAN administrator for help.

M    AMVER DEPARTURE MESSAGE

Composes AMVER departure message using the

information contained in the waypoint list.

(See D, A, and J)

M    AMVER POSITION REPORT

Composes AMVER position report based on the noon

position.  (You must enter the noon position

first).

PAGE 12

N    CREATE NEW WAYPOINT FILE

Used for making up new waypoint lists.  If you

already have a file for the route, it is easier to

load the existing file and use the editing routine

to make the necessary changes.  This saves you time

in not having to enter another chart list, etc.

O    FIND SLOW-DOWN WAYPOINTS

This routine is used when the ship is within a few

hours of arrival.  It calculates the time and

position to start slowing down in order to arrive

exactly on time.  Because it factors in the effects

of ship's displacement, wind and current, it

eliminates guesswork.

EXAMPLE #1 (Steamships):

Current speed is 20 knots, 78 RPM's.

Maneuvering speed is 55 RPM's.  Ship is 30 miles

from pilot station, and must arrive in exactly 2

hours.  At what time and in what position should

ship slow down to maneuvering speed in order to

arrive on time?

EXAMPLE #2 (Diesel ships):

Same as above, except ship must go through an

engine slow down program of 75 RPM for 15 minutes,

70 RPM for 10 minutes and 60 RPM for 20 minutes,

before slowing down to maneuvering speed.

In all cases the program will not allow a

maneuvering distance of less than 3 miles for

diesel powered ships or less than 4 miles for

steamships.  The reason for this is because by the

time the ship has actually arrived at the pilot

station it will have had to slow down from

maneuvering speed to dead slow, and the momentum

will carry a ship (depending on its displacement)

some distance actually slowing down.

Although the speed shown on the waypoint list is

the maneuvering speed, it works out that this loss

of speed is compensated for mathematically by the

momentum of the ship as it slowed down from full

sea speed to maneuvering speed.

PAGE 13

The net result is a smooth, professional and on

time arrival at the pilot station, without

resorting to excessive use of the throttle or other

maneuvers.

You will NOT be able to save a waypoint list that

has engine slow down positions calculated.  The

reason for this is the positions will vary,

depending upon the conditions of wind and sea

prevailing at the time, and there would be no reason

to save this for a later time.  The calculations

only take a minute to perform when needed.

P    POSITION REPORT            DD HH:MMZ  (1st POSITION)

DD HH:MMZ  (2nd POSITION)

For computing position report data.  Also required for

AMVER position reports and deviation messages.

Note that only the position and GMT are required to

make the position slip computations.

The subsequent entries are needed ONLY for a complete

position slip print out (6 to a page).  If you are going

to write out the slip by hand, then these additional

entries may be skipped.

After entering the time, the program will compute

and display a DR position as a reference to help

minimize errors.

Always remember to save the file after entering the

position so the program will be able to correctly

* * *  compute the next day's position slip.  UPTO 26 hours

is allowed between position slips.  This allows you

to enter an ARRIVAL position report and print it out.

Q    QUIT TO MAIN MENU

Returns to the MAIN MENU so you can exit the program.

R    RPM REQUIRED

The purpose of this routine is to calculate the

RPM's necessary from the present position in

order to arrive at a required time.  Somewhat

similar to the way the program calculates the

necessary speed, this routine also factors in the

effects of displacement, prevailing wind and sea

current, as well as the slowing down prior to

actual arrival, calculates the RPM's required and

displays this information on the waypoint list

along with the required speed.  Make sure that 2

fixes or noon positions have been entered PRIOR to

your present date and time.

PAGE 14

S    SAVE FILE

Saves the waypoint list to disk.  See file

naming convention in the "Description of program".

T    AUTO TRACK     ON/OFF

Similar to the DR position routine, except that

this routine continually displays current time, DR

position, distance to next waypoint, as well as

course to steer, current speed and ETA.  The program

continually updates this information every minute.

Operation is similar to that of many SatNav's and

Loran C, except that it can be made more accurate

and reliable, as you can choose only fixes that are

considered the most reliable and suitably spaced,

as well as celestial and other types of fixes.

On diesel-powered ships, if the engine slow down

points have been computed, then the screen will

also display the position and time of the next

scheduled engine slow-down.  When each position of

a speed change is reached the screen will flash the

next RPM, e.g., "DECREASE TO 70 RPM", and an

audible alarm will sound.  This routine runs in the

background and does not affect the operation of any

of the other routines.  However, at times when the

program may be doing computations it will not respond

immediately to a menu selection key pressed, but it

will eventually.  YOU NEED NOT PRESS THE KEY MORE

THAN ONCE.

PAGE 15

U    ENGINE SLOW-DOWN TIMER

(Diesel ships only).  This is a simple timer based

on the time periods and RPM's as stated in the

engine slow-down schedule.

Start the timer immediately after slowing down to

the first scheduled RPM.  Displayed on the screen

will be the current RPM, as well as the time, in

whole minutes, remaining until the next scheduled

speed reduction.

At time-out the timer will sound an audible alarm

and flash the next RPM's called for on the screen.

The alarm will sound for approximately 30 seconds

and shut off, or you may shut it off manually by

pressing the <SPACE BAR>.  It will then display the

current RPM and time remaining until the next

scheduled speed reduction, and so on, until it

reaches maneuvering speed.

V    AMVER DEVIATION MESSAGE

Composes AMVER deviation report. Includes revised

voyage track along with the latest noon position.

Note that you must enter the latest noon position

first.  Alternatively, you may enter the latest

fix just as if it were a noon position.

1.   AMVER routines will prompt for "REMARKS"

(AMVER "X" line).  If none, then hit <ENTER>

key.

2.   You may send AMVER messages to a printer, or

save them as an Email file.  When saving as

Email, the message will be preceded by the

Email telex number entered in the Program

set-up.

V    CONDENSE LIST FOR AMVER

This routine automatically combines all groups of

rhumb line legs into coastal legs of 80 miles or

less on the waypoint list.  Useful when sending

AMVER messages via morse code or telex.

PAGE 16

W    WAYPOINT LIST

Displays the waypoint list on the screen.  Waypoint

labels, chart lists and comments are omitted for

clarity.  Note: To see waypoint labels, use the "Edit"

routine from the waypoint list.  Use the "C - Chart

lists & comments" routine from the waypoint menu

to view those entries.  The following functions will

be described on the next page.

Edit Delete Insert WP-Arrival time MENU Create WP#   # Cursor # PgDn PgUp

�    �      �      �               �    �            �        � ���� ����

Y    DR POSITION

Calculates DR's at times entered.  Beginning at the

time entered, it will search backwards on the

waypoint list until it encounters either a fix or a

noon position.

If it finds a noon position first, then it will use

that position and the previous noon position, to

calculate the speed.  If it encounters a fix first

then it will use that fix as a reference point, and

search for a previous fix to calculate the speed.

If it doesn't find either a fix or a noon position,

then it will use the departure position and speed

to calculate the DR.

Note that because it always searches backward from

the time of the DR for the speed, it does

interpolate from fixes on the list entered later

than the time of the DR.

* * *  The only known bug in the program that I know of

(ALL others I detected I corrected) is that when you

enter a DR time desired EARLIER than that of DEPARTURE

the program reports that you are at the arrival port.

This is due to the poor way in which the time, in

particular, month and day, are calculated.  It only

looks at the month and not the day of the year.  Thus

the program gets confused when going to earlier times.

COMMON SENSE should tell you NOT to enter a time earlier

then DEPARTURE - the program does not! (yet)

Z    PRINT POSITION REPORT

Prints out a POSITION (NOON or ARRIVAL) slip six to a

page.

You must always remember to save a file after you have

made any entries.  If the save name does not agree with

the file name that was loaded, then the program will

query "Are you sure?" before saving, to minimize

mistakes.

PAGE 17

WAYPOINT LIST

Up to 15 waypoints can be displayed on one page.

If more waypoints exist then the final waypoint

will be displayed on the bottom one space below

the 15 waypoints shown.  This allows you to always

see the total distance and arrival time (GMT).  The

following functions are available as shown on the

bottom of the waypoint list screen display.  Not all

functions will always be available.  It depends upon

the number of waypoints that exist as described.

Edit Delete Insert WP-Arrival time MENU Create WP#   # Cursor # PgDn PgUp

�    �      �      �               �    �            �        � ���� ����

E    EDIT

For one or more waypoints.  Use to:

1.   Change waypoint data (latitude, longitude,

label, arrival time, and estimated speed and

track to next waypoint).

2.   Change names of arrival and departure ports

and zone description (incorporated as part of

waypoint #0).

3.   View waypoint labels without printing out the

list.

4.   "What if" spreadsheet planning.  For instance,

to answer the question "What is your best

ETA?"  Or you might want to substitute a rhumb

line track for a great circle track, or vice

verse, to see the what the difference in

distance and arrival time would be, etc.

The original file remains on the disk, unchanged.

D    DELETE

Deletes one or more waypoints from list.  The list,

if loaded from disk, still remains intact on the

disk until the file is resaved using either the same

or a new name.

* * *  Be careful NOT to delete a waypoint that is a noon

position, fix or has a required arrival time entered.

If you do delete one of these waypoints you should

re-enter the arrival time again so that the program

will properly work.

PAGE 18

I    INSERT (waypoint or FIX)

Inserts one or more waypoints or fixes into the

list.  Each time a fix is entered you get: (1)

speed check, (2) new course to steer, (3) revised

ETA or required speed.  These is a limit, at

present, of 61 waypoints.  A maximum of only

two noon positions are allowed for a file.  This is

automatically done.

W    WAYPOINT ARRIVAL TIME

Allows you to enter an arrival time at a specific

waypoint other than the final waypoint in the list.

This would be used, for example, when going through

the Inland Sea in Japan, where you must arrive at

the Moji Pilot Station enroute at a specific

time, due to the tide and vessel traffic.

To delete all remaining ETA's from a specific point

you must first have entered the waypoint arrival

time using this routine.  Then you can enter the

slash (/) key to delete all remaining ETA's.

M    MENU

Returns you to the Waypoint Menu.

C    CREATE WAYPOINT # xx

This function only shows up when less than 60

(0 - 59) waypoints exist in the list.  It allows

you to create the next waypoint number in the list.

# #  CURSOR

When more than 15 waypoints exist in a list you will

be allowed to scroll up or down the list one waypoint

at a time by using the up or down cursor keys.

PgUp PAGE UP

When more than 15 waypoints exist in a list you will

be allowed to scroll up the list one page (15 waypoints)

at a time by using the page up cursor key.

PgDn PAGE DOWN

When more than 15 waypoints exist in a list you will

be allowed to scroll down the list one page (15 waypoints)

at a time by using the page down cursor key.

PAGE 19

* * *  FINALLY, due to some program changes in my versions from

6/22/89 to 1/12/90 I have provided a new program which

allows you to UPDATE any of the old or existing data files

so that this new version will read and display the data

file properly.

UPDATE.EXE

A good indication if this program is needed is any periods

(.) showing up in the data fields when you view the waypoint

list OR all ETA's contain an asterisk (*).  Run the update

program on the data (.DAT) file and all will be okay.

This program will automatically be activated from the AMVER

program when needed!

In a future edition, hopefully a year from now, I will remove

the UPDATE routine and utilize that space to more efficiently

accomplish some of the tasks desired.

Over 200 hours have been spent playing with this program to

improve it as desired by deck officers and myself.  Each change

sometimes resulted in errors elsewhere in the program.  My

biggest difficulties were reorganizing the file structure to

allow compiling of the program and to continuously find space

to allow more changes.  In many cases it required the re-writing

of procedures to make the structure more efficient and smaller -

thus saving space for improvements.  If any of these changes

cause errors in mis-operations PLEASE let me know of the version

you are using and a detail explanation of the problem.  I will

try to correct it.  Thank you in advance.  When writing please

specify the version number (date on the main menu).  If you

are requesting an updated version please send a blank formatted

disk (I can handle any size disk 5-1/4" to 3-1/2" and any disk

capacity -360k to 1.44mb) and $2.00 to cover postage and handling.

Revised and compiled 3/22/90

by

Richard M. Plant

PAGE 20